Vivado Simulation Output

Detailed Explanation

Low-Latency Pipelined RISC-V 32-bit Processor (RV32I)

5-Stage Pipeline: IF → ID → EX → MEM → WB

Project: Low-Latency Pipelined RISC-V 32-bit Processor

Simulation Tool: Xilinx Vivado (Behavioral Simulation)

Total Simulation Time: 616 ns (~62 clock cycles at 100 MHz)

Table of Contents

1. Introduction

2. Processor Architecture Overview

3. Test Program — Complete Instruction Listing

4. Tcl Console Output — Detailed Explanation

5. Waveform Viewer — Detailed Explanation

6. Waveform Analysis — Observed Results

7. Complete Summary of Expected Results

8. Glossary of Terms

9. Troubleshooting Common Issues

# 1. Introduction

This document provides a comprehensive, beginner-friendly explanation of every simulation output produced when running the testbench (tb_riscv.v) for the 5-stage pipelined RV32I processor in Xilinx Vivado. It is written so that any team member — even one who has not worked on the Verilog code directly — can fully understand what the simulation does, what each output means, and how to verify that the processor is working correctly.

## 1.1 What Is This Simulation?

A simulation is a software-based test of our hardware design. Instead of loading the design onto a physical FPGA chip, Vivado’s built-in simulator executes the Verilog code on your computer. It processes every signal, every clock edge, and every logic gate — exactly as the real hardware would — but in a controlled environment where we can observe everything.

Think of it like a flight simulator: pilots test their skills without risking a real aircraft. Similarly, we test our processor design without needing physical hardware. The simulator lets us:

- See the value of every wire and register at every point in time

- Verify that instructions produce the correct results

- Confirm that hazard handling (forwarding, stalling, flushing) works without data corruption

- Debug problems by tracing signals cycle by cycle

## 1.2 Where Do Outputs Appear?

When you run the simulation in Vivado, outputs appear in two separate places:

| Output Location | Type | What It Shows | How to Find It |
| --- | --- | --- | --- |
| Tcl Console | Text-based | Printed messages from $display statements in the testbench: cycle traces, PASS/FAIL checks, final register values | Bottom panel in Vivado — scroll up to see all output |
| Waveform Viewer | Graphical | Signal waveforms plotted over time: clock, PCs, control signals, register values, ALU results, hazard signals | Center panel — opens automatically after simulation starts |

## 1.3 What Does the Test Program Test?

The testbench runs a 15-instruction test program that has been specifically crafted to exercise every major feature of the processor. Here is a summary of what each group of instructions tests:

| Feature Tested | Instructions Used | Why It Matters |
| --- | --- | --- |
| I-type immediate operations | addi x1, x0, 10 and addi x2, x0, 20 | Tests the most basic instruction type — loading constant values into registers. If these fail, nothing else will work. |
| R-type arithmetic (ADD) | add x3, x1, x2 | Tests register-to-register addition. Also tests EX→EX data forwarding since x1 and x2 were just written. |
| R-type arithmetic (SUB) | sub x4, x1, x2 | Tests subtraction and two’s complement negative number representation (−10 = 0xFFFFFFF6). |
| R-type logic (AND, OR, XOR) | and x5, or x6, xor x7 | Tests all three bitwise logic operations. Verifies the ALU’s logic unit and the ALU control decoder. |
| Store Word (SW) | sw x3, 0(x0) | Tests writing data from a register to data memory. Verifies the memory write path and address calculation. |
| Load Word (LW) | lw x8, 0(x0) | Tests reading data from memory into a register. Triggers a load-use hazard, testing the stall mechanism. |
| Load-use hazard + stall | lw x8 followed by nop then add x9 | Tests that the hazard detection unit correctly stalls the pipeline for 1 cycle when a loaded value is needed immediately. |
| MEM→EX forwarding | add x9, x8, x3 | Tests that data loaded from memory (x8) can be forwarded from the MEM/WB stage to the EX stage without waiting for write-back. |
| Branch (BEQ) + flush | beq x3, x3, +8 | Tests conditional branching. Since x3==x3 is always true, the branch is always taken. Tests that the pipeline correctly flushes incorrectly fetched instructions. |
| Branch target execution | addi x11, x0, 77 | Verifies that after the branch, execution correctly continues at the target address (0x34), not the sequential address. |
| Branch flush verification | addi x10, x0, 99 (SKIPPED) | This instruction should NEVER execute. If x10 != 0, the branch flush failed — a critical bug. |
| Halt (self-loop) | beq x0, x0, 0 | Creates an infinite loop to stop the processor. Without this, the PC would run into uninitialized memory. |

# 2. Processor Architecture Overview

Before diving into the simulation outputs, it is essential to understand the processor’s architecture. This section explains the 5-stage pipeline, the purpose of each stage, and the hazard handling mechanisms. If you already understand pipelining, you can skip to Section 3.

## 2.1 What Is a Pipeline?

A pipeline is a technique that allows a processor to work on multiple instructions simultaneously, similar to an assembly line in a factory. Instead of completing one instruction entirely before starting the next, the processor divides each instruction into stages and processes different instructions in different stages at the same time.

Analogy: Think of doing laundry. Without pipelining, you would wash load 1, dry load 1, fold load 1, then start load 2. With pipelining, while load 1 is drying, load 2 starts washing. While load 1 is being folded, load 2 is drying, and load 3 is washing. Each load still takes the same total time, but the throughput (loads completed per hour) is much higher.

## 2.2 The 5 Pipeline Stages

Our processor has 5 stages. Each instruction passes through all 5 stages in order, taking exactly 1 clock cycle per stage (so 5 cycles total per instruction). The stages are:

| Stage | Full Name | Source File | What Happens Here | Key Signals |
| --- | --- | --- | --- | --- |
| IF | Instruction Fetch | pc_reg.v, instr_mem.v | The Program Counter (PC) sends an address to instruction memory. The memory returns the 32-bit instruction at that address. The PC is then incremented by 4 (since each instruction is 4 bytes) to prepare for the next fetch. | pc_if, instr_if, pc_plus4_if |
| ID | Instruction Decode | control_unit.v, regfile.v, imm_gen.v | The fetched instruction is decoded. The opcode (bits [6:0]) is sent to the control unit, which generates all control signals. The register file reads the two source registers (rs1, rs2). The immediate generator extracts and sign-extends the immediate value. | instr_id, rs1_id, rs2_id, rdata1_id, rdata2_id, imm_id, all control signals |
| EX | Execute | alu.v, alu_ctrl.v, forward_unit.v | The ALU performs the operation specified by alu_ctrl (ADD, SUB, AND, OR, XOR, etc.). The forwarding unit checks if ALU inputs need to be bypassed from later stages. For branches, the condition is evaluated here. For loads/stores, the effective memory address is calculated. | alu_result_ex, forward_a, forward_b, branch_taken, flush_pipe |
| MEM | Memory Access | data_mem.v | Only active for load (LW) and store (SW) instructions. For SW, data is written to memory. For LW, data is read from memory. For all other instructions, this stage simply passes data through to WB. | mem_rdata_mem, alu_result_mem, memread_mem, memwrite_mem |
| WB | Write-Back | regfile.v (write port) | The final result (ALU result, memory data, or PC+4) is written back to the destination register (rd) in the register file. This completes the instruction. | wb_data, rd_wb, regwrite_wb |

Between each pair of stages, there is a pipeline register (also called a latch) that stores the instruction’s data and control signals as it moves from one stage to the next:

- IF/ID Register (if_id_reg.v): Stores the fetched instruction and its PC between IF and ID stages

- ID/EX Register (id_ex_reg.v): Stores decoded control signals, register values, and immediate between ID and EX stages

- EX/MEM Register (ex_mem_reg.v): Stores ALU result, store data, and control signals between EX and MEM stages

- MEM/WB Register (mem_wb_reg.v): Stores ALU result, loaded data, and write-back control between MEM and WB stages

Key Concept: Pipeline Registers Pipeline registers are the “walls” between stages. They capture data on every rising clock edge and hold it steady for the next stage to use. Without them, signals from different instructions would mix together and produce garbage. Each pipeline register updates on the rising edge of clk.

## 2.3 Pipeline Hazards — The Three Problems

Pipelining introduces three types of problems called hazards. Our processor handles all three:

### Problem 1: Data Hazard (RAW — Read After Write)

This happens when an instruction needs a value that a previous instruction hasn’t finished writing yet.

Example from our test program:

addi x1, x0, 10    ← writes x1 = 10 (result available after WB stage)

addi x2, x0, 20    ← writes x2 = 20

add  x3, x1, x2   ← reads x1 and x2 (but x1 hasn’t been written back yet!)

Solution: Forwarding Unit (forward_unit.v). Instead of waiting for the value to be written to the register file, the forwarding unit detects the dependency and routes the value directly from the pipeline register where it already exists. Two forwarding paths exist:

- EX→EX forwarding (forward = 2’b10): The value is available in the EX/MEM pipeline register (1 cycle old). The forwarding mux selects alu_result_mem.

- MEM→EX forwarding (forward = 2’b01): The value is available in the MEM/WB pipeline register (2 cycles old). The forwarding mux selects wb_data.

### Problem 2: Load-Use Hazard (Special Data Hazard)

Forwarding cannot solve every data hazard. When a load instruction (LW) is immediately followed by an instruction that uses the loaded value, the data is not available until the end of the MEM stage — one cycle too late for forwarding alone.

Example from our test program:

lw  x8, 0(x0)     ← loads x8 from memory (value available after MEM stage)

nop               ← this NOP exists because of the load-use hazard

add x9, x8, x3   ← needs x8 (now available via MEM→EX forwarding)

Solution: Hazard Detection Unit + 1-Cycle Stall (hazard_unit.v). When the hazard is detected, the pipeline is stalled for exactly 1 cycle: the PC holds its value, the IF/ID register replays the same instruction, and a NOP bubble is inserted into ID/EX. After the stall, the loaded value can be forwarded normally.

### Problem 3: Control Hazard (Branch Hazard)

When a branch instruction (like BEQ) is encountered, the processor doesn’t know whether the branch will be taken or not until it reaches the EX stage. By that time, the next 1–2 instructions have already been fetched into the pipeline. If the branch is taken, those instructions must be discarded.

Example from our test program:

beq  x3, x3, +8   ← branch decided in EX stage (taken: x3 == x3)

addi x10, x0, 99  ← already fetched into pipeline — must be FLUSHED

addi x11, x0, 77  ← this is the actual branch target at address 0x34

Solution: Pipeline Flush (riscv_top.v, line 314). When a branch is taken or a jump executes, flush_pipe is asserted. This clears the IF/ID and ID/EX pipeline registers (turning the incorrectly fetched instructions into NOPs) and redirects the PC to the branch target address.

# 3. Test Program — Complete Instruction Listing

The following table shows every instruction in the test program, stored in instruction memory (instr_mem.v, lines 37–51). Each row is one instruction. The Address column shows the byte address in memory, the Hex Code is the actual 32-bit machine code, and the remaining columns explain what the instruction does.

| Addr | Hex Code | Assembly | Operation | Expected Result & Notes |
| --- | --- | --- | --- | --- |
| 0x00 | 0x00A00093 | addi x1, x0, 10 | x1 = 0 + 10 | x1 = 10. First instruction executed after reset. |
| 0x04 | 0x01400113 | addi x2, x0, 20 | x2 = 0 + 20 | x2 = 20. Sets up second operand for subsequent tests. |
| 0x08 | 0x002081B3 | add x3, x1, x2 | x3 = x1 + x2 | x3 = 30. Tests EX→EX forwarding — x1, x2 not yet in register file. |
| 0x0C | 0x40208233 | sub x4, x1, x2 | x4 = x1 − x2 | x4 = −10 (0xFFFFFFF6). Tests SUB and two’s complement. |
| 0x10 | 0x0020F2B3 | and x5, x1, x2 | x5 = x1 & x2 | x5 = 0. Binary: 01010 AND 10100 = 00000. |
| 0x14 | 0x0020E333 | or x6, x1, x2 | x6 = x1 ¦ x2 | x6 = 30. Binary: 01010 OR 10100 = 11110 = 30. |
| 0x18 | 0x0020C3B3 | xor x7, x1, x2 | x7 = x1 ^ x2 | x7 = 30. Binary: 01010 XOR 10100 = 11110 = 30. |
| 0x1C | 0x00302023 | sw x3, 0(x0) | mem[0] = x3 | Stores 30 at memory address 0. Tests memory write path. |
| 0x20 | 0x00002403 | lw x8, 0(x0) | x8 = mem[0] | x8 = 30. Triggers load-use hazard (stall expected). |
| 0x24 | 0x00000013 | nop | No operation | Pipeline bubble — allows LW result to propagate. |
| 0x28 | 0x003404B3 | add x9, x8, x3 | x9 = x8 + x3 | x9 = 60. Tests MEM→EX forwarding for x8. |
| 0x2C | 0x00318463 | beq x3, x3, +8 | if x3==x3, jump +8 | Always taken → jumps to 0x34. Tests branch + flush. |
| 0x30 | 0x06300513 | addi x10, x0, 99 | x10 = 99 | SKIPPED — flushed by branch. x10 must stay 0. |
| 0x34 | 0x04D00593 | addi x11, x0, 77 | x11 = 0 + 77 | x11 = 77. Branch target — execution resumes here. |
| 0x38 | 0x00000063 | beq x0, x0, 0 | Branch to self | Halt — infinite self-loop stops execution. |

Color Key for Table Above Blue rows = normal instructions Red row (0x30) = instruction that should be SKIPPED due to branch flush Green row (0x34) = branch target where execution should resume

# 4. Tcl Console Output — Detailed Explanation

The Tcl Console is the text output panel at the bottom of the Vivado window. When the simulation runs, the testbench’s $display statements print text here — similar to printf() in C or console.log() in JavaScript. The output is divided into five distinct sections, each testing a different aspect of the processor.

How to read the Tcl Console: After the simulation completes, scroll up in the Tcl Console to see all output. The output starts with "=== RESET RELEASED — Simulation START ===", followed by 60 lines of cycle traces, then the verification sections, and finally "=== Simulation COMPLETE ===".

## 4.1 Cycle-by-Cycle Trace

Source: tb_riscv.v, lines 69–80 (the always @(posedge clk) block)

When it prints: Every single clock cycle throughout the entire simulation

Purpose: Lets you see the pipeline state at each moment, helping you trace exactly what the processor is doing

Each line has this format:

Cycle 1 | PC_IF=0x00000000 | PC_ID=0x00000000 | PC_EX=0x00000000 | stall=0 | flush=0

Let’s break down each field in detail:

### 4.1.1 Cycle Number

This is a simple counter that increments by 1 on every clock cycle (variable cycle_cnt, line 69). It starts at 0 during reset and reaches about 62 by the end. This number appears in the leftmost position and helps you identify specific cycles when debugging. For example, if a register value is wrong, you can look at the cycle where the instruction should have executed.

### 4.1.2 PC_IF — Instruction Fetch Program Counter

What it is: The address of the instruction currently being fetched from instruction memory in the IF (Instruction Fetch) stage.

Source signal: DUT.pc_if (driven by pc_reg.v)

Normal behavior: Increments by 4 every cycle (0x00000000, 0x00000004, 0x00000008, 0x0000000C, ...) because each instruction is exactly 4 bytes long.

Abnormal behaviors to watch for:

- Holds steady for one cycle — this means stall=1 (a load-use hazard was detected). The PC is frozen to replay the same fetch.

- Jumps to a non-sequential address (e.g., suddenly shows 0x00000034 instead of 0x00000030) — this means a branch was taken and flush=1. The PC was redirected to the branch target.

- Stays at 0x00000038 forever — this is the halt instruction (beq x0, x0, 0), which branches to itself, creating an infinite loop. This is expected at the end.

### 4.1.3 PC_ID — Instruction Decode Program Counter

What it is: The address of the instruction currently being decoded in the ID (Instruction Decode) stage. At this point, the control unit is determining what type of instruction it is, and the register file is reading the source register values.

Source signal: DUT.pc_id (output of IF/ID pipeline register)

Key insight: PC_ID is always one cycle behind PC_IF. When you see PC_IF=0x08, PC_ID will still show the address of the previous instruction (0x04). This is because the IF/ID register captures PC_IF on the clock edge and holds it for the ID stage.

### 4.1.4 PC_EX — Execute Program Counter

What it is: The address of the instruction currently being executed in the EX (Execute) stage. This is where the ALU performs calculations, the forwarding unit makes bypassing decisions, and branch conditions are evaluated.

Source signal: DUT.pc_ex (output of ID/EX pipeline register)

Key insight: PC_EX is two cycles behind PC_IF. This 2-cycle delay is a direct consequence of having two pipeline registers (IF/ID and ID/EX) between the IF and EX stages.

Understanding the Pipeline Delay When you see: Cycle 5 ¦ PC_IF=0x10 ¦ PC_ID=0x0C ¦ PC_EX=0x08 This means: • The IF stage is fetching the instruction at address 0x10 (and x5, x1, x2) • The ID stage is decoding the instruction at address 0x0C (sub x4, x1, x2) • The EX stage is executing the instruction at address 0x08 (add x3, x1, x2) All three instructions are active simultaneously — this is pipelining in action!

### 4.1.5 stall Signal

What it is: A 1-bit signal from the hazard detection unit (hazard_unit.v) that indicates a load-use hazard has been detected.

Source signal: DUT.stall (output of hazard_unit.v)

When it is 1: When the instruction currently in the EX stage is a load (LW) and its destination register is the same as a source register of the instruction currently in the ID stage. In other words, the next instruction needs data that hasn’t been loaded from memory yet.

The exact hardware condition (hazard_unit.v, lines 22–24):

stall = id_ex_memread &&

((id_ex_rd == if_id_rs1) || (id_ex_rd == if_id_rs2)) &&

(id_ex_rd != 5’b0)

What happens when stall = 1 (three simultaneous actions):

- PC register freezes: The pc_reg.v module (line 16: else if (!stall)) does not update the PC. The same instruction address stays on the instruction memory input, so the same instruction will be fetched again next cycle.

- IF/ID register freezes: The if_id_reg.v module (line 23: else if (!stall)) does not update. The same instruction stays in the ID stage for one extra cycle, waiting for the loaded data to become available.

- ID/EX register is flushed: The id_ex_reg.v module receives flush=1 (because id_ex_flush = stall || flush_pipe in riscv_top.v, line 196). All control signals in ID/EX are cleared to zero, creating a NOP bubble — an instruction that does nothing. This prevents the stalled instruction from executing with stale data.

In our test program: The stall signal should go to 1 for exactly one cycle when the LW at address 0x20 is in the EX stage and the following instruction (NOP at 0x24 or the instruction after it) is in the ID stage with a matching register dependency.

### 4.1.6 flush Signal

What it is: A 1-bit signal that indicates a branch was taken or a jump was executed. The pipeline must discard (flush) the incorrectly fetched instructions.

Source signal: DUT.flush_pipe (riscv_top.v, line 314)

The exact hardware condition:

flush_pipe = branch_taken || jump_taken

where: branch_taken = branch_ex && branch_cond

jump_taken   = jal_ex   || jalr_ex

What happens when flush = 1 (three simultaneous actions):

- PC jumps to target: The pc_next mux (riscv_top.v, line 122) selects jump_target instead of pc_plus4_if. For our BEQ at 0x2C: jump_target = pc_ex + imm_ex = 0x2C + 8 = 0x34.

- IF/ID register is cleared: The if_id_reg.v module (line 20: if (rst || flush)) sets instruction and PC to 0. The instruction that was being fetched (at 0x30) is replaced with a NOP.

- ID/EX register is cleared: The id_ex_reg.v module (line 62: if (rst || flush)) sets all control signals to 0. The instruction that was being decoded is replaced with a NOP bubble.

In our test program: The flush signal should go to 1 when the BEQ at address 0x2C reaches the EX stage and the branch condition evaluates to true. It should also go to 1 on every cycle during the halt loop (since beq x0, x0, 0 is a taken branch every time).

## 4.2 Register File Check

Source: tb_riscv.v, lines 106–117

When it prints: After 60 clock cycles of execution (enough for all instructions to complete)

Purpose: Reads the final value of each register directly from the hardware register file and compares it against the expected value

The expected console output looks like this:

========== REGISTER FILE CHECK ==========

x1  = 10  (expected 10)  PASS

x2  = 20  (expected 20)  PASS

x3  = 30  (expected 30)  PASS

x4  = -10  (expected -10) PASS

x5  = 0   (expected 0)   PASS

x6  = 30  (expected 30)  PASS

x7  = 30  (expected 30)  PASS

x8  = 30  (expected 30)  PASS

x9  = 60  (expected 60)  PASS

x10 = 0   (expected 0)   PASS

x11 = 77  (expected 77)  PASS

How it works internally: The testbench uses a Verilog task called read_reg (line 47) that accesses the register file using the hierarchical path DUT.RF.regs[reg_num]. This is a simulation-only feature — it lets the testbench peek directly inside the hardware module without needing any output ports. In real hardware on an FPGA, you cannot do this; but in simulation, it is invaluable for verification.

Detailed explanation of each register:

| Reg | Value | Why This Value (Step-by-Step Explanation) | Tests | Verdict |
| --- | --- | --- | --- | --- |
| x1 | 10 | Instruction: addi x1, x0, 10. Takes the value of register x0 (which is always 0 in RISC-V — hardwired to zero) and adds the immediate value 10. Result: 0 + 10 = 10. This is stored into register x1. | I-type ALU | PASS |
| x2 | 20 | Instruction: addi x2, x0, 20. Same operation as x1 but with immediate value 20. Result: 0 + 20 = 20. Stored into x2. | I-type ALU | PASS |
| x3 | 30 | Instruction: add x3, x1, x2. Reads x1 (=10) and x2 (=20) and adds them. Result: 10 + 20 = 30. Note: x1 and x2 may not yet be in the register file when this instruction executes — the forwarding unit provides their values via EX→EX forwarding. | R-type ADD, EX→EX fwd | PASS |
| x4 | −10 | Instruction: sub x4, x1, x2. Subtracts: 10 − 20 = −10. In 32-bit two’s complement, −10 is represented as 0xFFFFFFF6 (binary: 11111111111111111111111111110110). The testbench displays this as −10 using $signed(). | R-type SUB | PASS |
| x5 | 0 | Instruction: and x5, x1, x2. Bitwise AND: 10 in binary is 01010, 20 in binary is 10100. AND-ing each bit position: 0&1=0, 1&0=0, 0&1=0, 1&0=0, 0&0=0. Result: 00000 = 0. No bit positions have 1 in both operands. | R-type AND | PASS |
| x6 | 30 | Instruction: or x6, x1, x2. Bitwise OR: 01010 OR 10100 = 11110. Converting 11110 from binary to decimal: 16+8+4+2 = 30. | R-type OR | PASS |
| x7 | 30 | Instruction: xor x7, x1, x2. Bitwise XOR (exclusive OR): 01010 XOR 10100 = 11110 = 30. XOR produces 1 when bits differ. Since 10 and 20 have no overlapping bits, XOR gives the same result as OR here. | R-type XOR | PASS |
| x8 | 30 | Instruction: lw x8, 0(x0). Loads a 32-bit word from data memory at address 0 (base register x0=0, offset=0). Address 0 contains the value 30, which was stored there earlier by the SW instruction. This tests the complete store→load memory path. | LW, memory path | PASS |
| x9 | 60 | Instruction: add x9, x8, x3. Adds x8 (=30, loaded from memory) + x3 (=30) = 60. When this instruction is in EX, x8’s value comes via MEM→EX forwarding from the MEM/WB pipeline register, because the LW completed 2 cycles earlier. | ADD, MEM→EX fwd | PASS |
| x10 | 0 | Instruction: addi x10, x0, 99 was NEVER EXECUTED. The branch at 0x2C was taken, flushing this instruction from the pipeline before it could reach the EX stage. Since x10 was initialized to 0 at reset and was never written, it remains 0. If x10 ≠ 0, the branch flush mechanism has a bug. | Branch flush | PASS |
| x11 | 77 | Instruction: addi x11, x0, 77. This is at address 0x34, the branch target. After the BEQ at 0x2C is taken, the PC jumps to 0x34, and this instruction executes normally. Result: 0 + 77 = 77. Confirms the branch landed at the correct target. | Branch target | PASS |

Critical Requirement ALL 11 register checks must show PASS. Even a single FAIL indicates a functional bug in the processor. The most common causes of failure are: • Incorrect control signal generation in control_unit.v • Wrong ALU operation selected by alu_ctrl.v • Forwarding unit not detecting a dependency (forward_unit.v) • Pipeline flush not clearing instructions properly (riscv_top.v)

## 4.3 Data Memory Check

Source: tb_riscv.v, lines 119–121

Purpose: Verifies that the Store Word (SW) instruction correctly wrote data to memory

Expected console output:

========== DATA MEMORY CHECK ============

dmem[0] = 30  (expected 30)  PASS

How the read_dmem task works (tb_riscv.v, line 57): The task reads 4 individual bytes from the data memory array using hierarchical paths (DUT.DMEM.mem[addr], DUT.DMEM.mem[addr+1], etc.) and concatenates them into a 32-bit word in little-endian byte order (least significant byte at the lowest address). This matches the RISC-V specification.

Step-by-step trace of how the value 30 got into memory:

- Step 1 (IF): The instruction sw x3, 0(x0) at address 0x1C is fetched from instruction memory.

- Step 2 (ID): The control unit decodes opcode 0100011 (OP_STORE) and asserts memwrite=1, alusrc=1 (use immediate for address). The register file reads x3=30 (store data) and x0=0 (base address). The immediate generator extracts offset=0.

- Step 3 (EX): The ALU computes the effective address: base (x0=0) + offset (0) = 0. The store data (x3=30) is passed through to the EX/MEM register.

- Step 4 (MEM): Since memwrite=1, data_mem.v writes 30 to byte addresses 0–3: mem[0]=0x1E, mem[1]=0x00, mem[2]=0x00, mem[3]=0x00 (little-endian representation of 30).

- Step 5: The read_dmem task reconstructs: {mem[3], mem[2], mem[1], mem[0]} = {0x00, 0x00, 0x00, 0x1E} = 0x0000001E = 30. PASS.

## 4.4 Forwarding Unit Check

Source: tb_riscv.v, lines 123–125

Purpose: Verifies that the forwarding unit correctly resolves Read-After-Write (RAW) data hazards

Expected console output:

========== FORWARDING UNIT CHECK ========

EX->EX forward test (x3=x1+x2): PASS

MEM->EX forward test (x9=x8+x3): PASS

### 4.4.1 EX→EX Forwarding Test (Detailed Walkthrough)

This tests the scenario where an instruction needs a value computed just 1 cycle earlier.

The hazard situation:

Cycle N:   addi x2, x0, 20   ← in EX stage, computing x2 = 20

Cycle N:   add  x3, x1, x2   ← in ID stage, needs x2 (not written yet!)

Cycle N+1: addi x2 moves to MEM stage, x2’s result is in EX/MEM register

Cycle N+1: add  x3 moves to EX stage, needs x2 NOW for ALU input B

How forwarding solves it: In Cycle N+1, the forwarding unit (forward_unit.v) checks: Is the instruction that just left EX (now in EX/MEM register) writing to a register (ex_mem_regwrite=1) that the current EX instruction reads (ex_mem_rd == id_ex_rs2)? Yes! ex_mem_rd=x2 matches id_ex_rs2=x2. So forward_b = 2’b10, and the forwarding mux (riscv_top.v, line 270) routes alu_result_mem directly to the ALU’s B input, bypassing the register file entirely.

PASS condition: DUT.RF.regs[3] == 30 (if forwarding failed, x3 would contain a wrong value like 0+20=20 or 10+0=10).

### 4.4.2 MEM→EX Forwarding Test (Detailed Walkthrough)

This tests the scenario where an instruction needs a value from 2 cycles earlier (specifically a loaded value).

The hazard situation:

Cycle N:   lw   x8, 0(x0)     ← in MEM stage, reading memory into x8

Cycle N:   nop                 ← in EX stage (pipeline bubble)

Cycle N:   add  x9, x8, x3    ← in ID stage, needs x8 (not written yet!)

Cycle N+1: lw   x8 moves to WB, x8’s value is in MEM/WB register

Cycle N+1: add  x9 moves to EX, needs x8 NOW for ALU input A

How forwarding solves it: In Cycle N+1, the forwarding unit checks: Is the instruction in MEM/WB writing to a register (mem_wb_regwrite=1) that the current EX instruction reads (mem_wb_rd == id_ex_rs1)? Yes! mem_wb_rd=x8 matches id_ex_rs1=x8. So forward_a = 2’b01, and the forwarding mux (riscv_top.v, line 267) routes wb_data to the ALU’s A input.

PASS condition: DUT.RF.regs[9] == 60 (30 + 30).

Complete Forwarding Mux Truth Table:

| forward_a / forward_b Value | Source | Data Routed to ALU | When This Happens |
| --- | --- | --- | --- |
| 2’b00 (no forward) | Register file output | rdata1_ex or rdata2_ex | No hazard detected — register file has the correct value |
| 2’b10 (EX→EX) | EX/MEM pipeline register | alu_result_mem | Previous instruction (1 cycle ago) wrote to a register we need. EX/MEM has priority. |
| 2’b01 (MEM→EX) | MEM/WB pipeline register | wb_data | Instruction from 2 cycles ago wrote to a register we need. Only used when EX/MEM doesn’t match. |

## 4.5 Branch Flush Check

Source: tb_riscv.v, lines 127–129

Purpose: Verifies that the pipeline correctly discards instructions fetched after a taken branch

Expected console output:

========== BRANCH FLUSH CHECK ===========

x10 should be 0 (addi x10,x0,99 was skipped): PASS

Complete cycle-by-cycle walkthrough of the branch:

| Cycle | IF Stage | ID Stage | EX Stage | What Happens |
| --- | --- | --- | --- | --- |
| N | Fetching 0x30 (addi x10) | Decoding 0x2C (beq) | — | Branch instruction enters ID stage |
| N+1 | Fetching 0x34 (addi x11) | 0x30 enters ID | 0x2C (beq) enters EX | Branch condition evaluated: x3==x3 → TRUE. flush_pipe=1! |
| N+1 | — | — | — | PC redirected to 0x34 (branch target = 0x2C + 8) |
| N+2 | Fetching 0x34 (correct target) | IF/ID FLUSHED to NOP | ID/EX FLUSHED to NOP | Instructions from 0x30 and 0x34 (old fetch) are discarded |
| N+3 | Fetching 0x38 (halt) | 0x34 (addi x11) in ID | NOP bubble | Normal execution resumes from branch target |

Why x10 stays 0: The instruction addi x10, x0, 99 at address 0x30 was fetched into the IF stage during cycle N. It moved to the ID stage during cycle N+1. But at the same time, the branch at 0x2C was evaluated in EX and determined to be taken. The flush_pipe signal cleared the IF/ID and ID/EX registers at the end of cycle N+1. This means addi x10 never reached the EX stage and therefore never executed. Register x10, which was initialized to 0 at reset and never written, remains 0.

PASS confirms: (1) Branch condition was correctly evaluated, (2) flush_pipe was correctly asserted, (3) IF/ID and ID/EX registers were correctly cleared, (4) PC was correctly redirected to 0x34, (5) No incorrectly fetched instruction corrupted the register file.

# 5. Waveform Viewer — Detailed Explanation

The Waveform Viewer is the graphical simulation output window that opens in the center of the Vivado workspace. Unlike the Tcl Console (which shows text), the Waveform Viewer shows signals plotted as waveforms over time, similar to an oscilloscope. This is the most powerful debugging tool available — it lets you see the exact state of every signal at every nanosecond.

## 5.1 Step-by-Step: How to Open and Use the Waveform Viewer

| Step | Action | What You See | Notes |
| --- | --- | --- | --- |
| 1 | In the Flow Navigator (left panel), click SIMULATION → Run Simulation → Run Behavioral Simulation | Vivado compiles all .v files and opens the Simulation workspace | Compilation messages appear in the Tcl Console. If there are errors, fix them first. |
| 2 | The Waveform Viewer opens automatically in the center panel | A waveform window with some default signals | The file tb_riscv_behav.wcfg is loaded automatically with pre-configured signals. |
| 3 | Click the "Run All" button (solid triangle icon) or press Shift+F5 | The simulation runs until $finish is reached | You should see the Tcl Console output scrolling. The simulation takes about 616 ns. |
| 4 | Click the "Zoom Fit" button (magnifying glass with arrows) or press Ctrl+0 | The entire simulation timeline fits in the window | You can now see all waveforms from 0 ns to ~616 ns. |
| 5 | To add more signals: In the Scope panel (left), expand tb_riscv → DUT → [submodule] | The Objects panel shows all signals in that module | Drag signals from Objects into the waveform window. |
| 6 | Right-click a signal → Radix → choose Unsigned Decimal or Hexadecimal | Signal values displayed in your chosen format | Hex is useful for addresses (pc_if), Decimal for data (register values). |
| 7 | Click anywhere on the waveform to place a time cursor | A yellow vertical line appears showing the exact time | The signal values at that time are shown in the Name/Value columns on the left. |

## 5.2 Understanding the Scope Panel (Module Hierarchy)

The Scope panel on the left shows the design hierarchy. Here is the complete tree with explanations:

tb_riscv                    ← Top-level testbench (contains clk, rst, cycle_cnt)

└─ DUT (riscv_top)         ← Device Under Test — the actual processor

├─ PC   (pc_reg)       ← Program Counter register

├─ IMEM (instr_mem)    ← Instruction Memory (ROM with test program)

├─ IF_ID (if_id_reg)   ← IF/ID pipeline register

├─ CTRL (control_unit) ← Main instruction decoder

├─ RF   (regfile)      ← Register File (32 registers, x0–x31)

├─ IGN  (imm_gen)      ← Immediate Generator

├─ HDU  (hazard_unit)  ← Hazard Detection Unit (load-use stalls)

├─ ID_EX (id_ex_reg)   ← ID/EX pipeline register

├─ FU   (forward_unit) ← Forwarding Unit (data bypass)

├─ ALUCTL (alu_ctrl)   ← ALU Control decoder

├─ ALU  (alu)          ← Arithmetic Logic Unit

├─ EX_MEM (ex_mem_reg) ← EX/MEM pipeline register

├─ DMEM (data_mem)     ← Data Memory (RAM for load/store)

└─ MEM_WB (mem_wb_reg) ← MEM/WB pipeline register

## 5.3 Key Signals — What to Look At and Why

### 5.3.1 Clock and Reset

clk — A perfect square wave with a 10 ns period (5 ns high, 5 ns low), giving a 100 MHz clock. This is the heartbeat of the processor. Every signal changes on the rising edge (0→1 transition) of this clock. Generated by: always #5 clk = ~clk in tb_riscv.v.

rst — Goes high at time 0, stays high for approximately 20 ns (2 clock cycles), then drops to 0. While rst=1, every pipeline register is cleared to zero, the PC is reset to 0x00000000, and all register file entries are set to 0. The processor starts executing only after rst goes low.

### 5.3.2 Program Counters (pc_if, pc_id, pc_ex)

These three signals are the most important for understanding program flow:

| Signal | Pipeline Stage | What It Shows | Normal Pattern | Abnormal Pattern (and What It Means) |
| --- | --- | --- | --- | --- |
| pc_if | IF (Fetch) | Address being fetched from instruction memory | Increments by 4 every cycle: 0x00, 0x04, 0x08, ... | Holds value = stall. Jumps to non-sequential address = branch/jump taken. |
| pc_id | ID (Decode) | Address being decoded; 1 cycle behind pc_if | Follows pc_if with 1-cycle delay | Shows 0x00000000 after flush (NOP injected by clearing IF/ID register) |
| pc_ex | EX (Execute) | Address being executed; 2 cycles behind pc_if | Follows pc_id with 1-cycle delay | Shows 0x00000000 after flush (NOP injected by clearing ID/EX register) |

### 5.3.3 Hazard Signals (stall, flush_pipe)

stall — Normally 0. Pulses to 1 for exactly 1 cycle when a load-use hazard is detected. In our program, this happens when the LW at 0x20 is in EX and the next instruction needs the loaded register. During stall=1, you will see pc_if freeze (hold its value) and the ID stage replay the same instruction.

flush_pipe — Normally 0. Goes to 1 when a branch is taken. In our program, this happens when the BEQ at 0x2C is in EX and the condition is true (x3==x3). After the BEQ, flush_pipe also goes to 1 on every cycle during the halt loop (beq x0, x0, 0 is always taken). During flush=1, you will see the IF/ID and ID/EX registers cleared.

### 5.3.4 Forwarding Signals (forward_a, forward_b)

These 2-bit signals are found in DUT → FU (forward_unit):

- 00 = No forwarding needed. The register file has the correct value.

- 10 = EX→EX forwarding. The value comes from the EX/MEM pipeline register (1 cycle old). This is the highest priority.

- 01 = MEM→EX forwarding. The value comes from the MEM/WB pipeline register (2 cycles old).

Watch for forward_a and forward_b changing to 10 or 01 during the ADD instructions that test forwarding.

### 5.3.5 ALU Signals (a, b, alu_ctrl, result)

Found in DUT → ALU:

| alu_ctrl Value | Operation | Verilog Code | Example in Test Program |
| --- | --- | --- | --- |
| 4’b0000 | ADD (a + b) | result = a + b | addi x1,x0,10: a=0, b=10, result=10 |
| 4’b0001 | SUB (a − b) | result = a - b | sub x4,x1,x2: a=10, b=20, result=−10 |
| 4’b0010 | AND (a & b) | result = a & b | and x5,x1,x2: a=10, b=20, result=0 |
| 4’b0011 | OR (a ¦ b) | result = a ¦ b | or x6,x1,x2: a=10, b=20, result=30 |
| 4’b0100 | XOR (a ^ b) | result = a ^ b | xor x7,x1,x2: a=10, b=20, result=30 |

### 5.3.6 Register File (regs[1]–regs[11])

Found in DUT → RF. These show the actual hardware register values. Watch them transition from 0 to their final values. Key observations:

- All registers start at 0 during reset.

- The first register write (x1=10) appears approximately 50 ns after reset release (~70 ns from time 0). This is exactly 5 clock cycles — the latency of the 5-stage pipeline.

- After the pipeline fills, one new register value appears every 10 ns (1 clock cycle).

- Register x10 should never change from 0 (the branch flush test).

- Set the radix to Unsigned Decimal for easier reading.

### 5.3.7 Data Memory Signals

Found in DUT → DMEM:

- memwrite — Goes to 1 for exactly one cycle when the SW instruction is in the MEM stage. During this cycle, addr=0x00000000 and wdata=0x0000001E (30).

- memread — Goes to 1 for exactly one cycle when the LW instruction is in the MEM stage. During this cycle, addr=0x00000000 and rdata=0x0000001E (30).

- These signals should each pulse high for only one clock cycle each.

### 5.3.8 Control Signals

Found in DUT → CTRL. These change every cycle based on the instruction being decoded:

| Instruction Type | regwrite | memread | memwrite | alusrc | branch | aluop |
| --- | --- | --- | --- | --- | --- | --- |
| R-type (add, sub, and, or, xor) | 1 | 0 | 0 | 0 | 0 | 10 |
| I-type ALU (addi) | 1 | 0 | 0 | 1 | 0 | 10 |
| Load (lw) | 1 | 1 | 0 | 1 | 0 | 00 |
| Store (sw) | 0 | 0 | 1 | 1 | 0 | 00 |
| Branch (beq) | 0 | 0 | 0 | 0 | 1 | 01 |
| NOP (after flush/stall) | 0 | 0 | 0 | 0 | 0 | 00 |

## 5.4 Complete Waveform Timeline

The following table provides a detailed timeline of what happens at each phase of the simulation:

| Time | Event | Detailed Description of What to Observe in the Waveform |
| --- | --- | --- |
| 0–20 ns | Reset | rst=1 (high). All signals are at 0 or X. clk is running but no instructions execute. All pipeline registers output 0. pc_if stays at 0x00000000. Register file entries are all 0. |
| ~20 ns | Reset released | rst transitions from 1 to 0. On the next rising clock edge, pc_if starts incrementing. The first instruction (addi x1, x0, 10) is fetched from address 0x00. The pipeline begins filling. |
| 20–70 ns | Pipeline filling | pc_if increments by 4 each cycle: 0x00, 0x04, 0x08, 0x0C, 0x10. The first instruction moves through IF→ID→EX→MEM→WB. No results written yet because no instruction has reached WB. |
| ~70 ns | First WB | The first instruction (addi x1) completes WB. regs[1] changes from 0 to 10. regwrite_wb=1 for this cycle. This is the first visible register change. |
| 70–120 ns | Steady execution | One instruction completes per cycle. Registers x2 through x7 get written in sequence. ALU result signal shows: 20, 30, -10, 0, 30, 30. forward_a/forward_b may pulse to 2’b10 during add/sub/and/or/xor. |
| ~120 ns | SW executes | The SW instruction reaches MEM stage. memwrite_mem=1 for one cycle. addr shows 0x00000000, wdata shows 0x0000001E (30). Data is written to memory bytes mem[0]–mem[3]. |
| ~130 ns | LW + stall | The LW instruction reaches EX/MEM. stall may pulse to 1 for one cycle if a dependency exists. memread_mem=1 for one cycle. rdata shows 0x0000001E (30). pc_if holds its value during the stall. |
| ~150 ns | x9 computed | add x9, x8, x3 executes. forward_a=2’b01 (MEM→EX forwarding for x8). ALU computes 30+30=60. regs[9] gets written to 60 when this instruction reaches WB. |
| ~160 ns | BEQ taken | beq x3, x3, +8 reaches EX. branch_taken=1, flush_pipe=1. PC jumps from sequential address to 0x34. IF/ID and ID/EX registers are cleared to 0 (NOP bubbles appear in pc_id and pc_ex). |
| ~170 ns | Branch target | addi x11, x0, 77 is fetched from 0x34 and begins flowing through the pipeline. Two NOP bubbles from the flush pass through EX and MEM. |
| ~180+ ns | Halt loop | beq x0, x0, 0 at 0x38 branches to itself every cycle. pc_if stays at 0x38. flush_pipe=1 on every cycle (taken branch). All register values are stable. The processor is effectively halted. |
| ~616 ns | $finish | After 60 cycles of execution, the testbench runs verification checks (register file, memory, forwarding, branch flush) and calls $finish. The simulation stops. |

# 6. Waveform Analysis — Observed Simulation Results

This section describes the actual waveforms observed after running the simulation. The simulation ran for a total of 616 ns (~62 clock cycles).

## 6.1 Top-Level Testbench Waveform (tb_riscv_behav.wcfg)

The waveform configuration file tb_riscv_behav.wcfg loads automatically and displays the following signals:

| Signal Name | Type | Final Value | Waveform Color | Detailed Interpretation |
| --- | --- | --- | --- | --- |
| clk | 1-bit | 1 | Green (valid) | Perfect square wave, 10 ns period, 100 MHz. 62 complete cycles from start to $finish. Confirms the clock generator operated correctly. |
| rst | 1-bit | 0 | Green (valid) | High for first ~20 ns (2 cycles), then permanently low. All pipeline registers were properly reset. Execution began on the first rising edge after rst→0. |
| cycle_cnt[31:0] | 32-bit bus | 0x0000003E (62) | Green (valid) | Steady ramp from 0 to 62. Each step is one clock cycle. Confirms 2 reset cycles + 60 execution cycles = 62 total. |
| i[31:0] | 32-bit integer | XXXXXXXX | Red (unknown) | This is the loop variable from the testbench. It shows X (red) for most of the simulation because it is only assigned during the verification phase at ~600+ ns. Red in Vivado = unknown/uninitialized. |
| r_val[31:0] | 32-bit reg | 0x0000004D (77) | Red → Green | X until verification phase, then cycles through: 10, 20, 30, 0xFFFFFFF6, 0, 30, 30, 30, 60, 0, 77. Final value 77 = x11 = LAST register checked. Confirms x11=77 is correct. |
| dmem_val[31:0] | 32-bit reg | 0x0000001E (30) | Red → Green | X until memory check, then stabilizes at 30. 0x1E = 30 decimal. Confirms data memory at address 0 holds the correct value written by SW. |

## 6.2 Register File Waveform (Bit-Level View)

The second waveform view shows individual register file bits [16] down to [0]:

| Register Bits | Waveform | Explanation |
| --- | --- | --- |
| [16]–[12] | Flat at 0 (green) | Registers x12–x16 are never written by the test program. They stay at their reset value of 0. Confirms no spurious writes occurred. |
| [11] | Transitions to non-zero | Register x11 gets written to 77 (0x4D) when the branch target instruction (addi x11, x0, 77) completes WB. |
| [10] | Flat at 0 (green) | CRITICAL: Register x10 is NEVER written. This is the branch flush test — addi x10, x0, 99 was flushed and never executed. x10=0 confirms the flush worked. |
| [9] | Transitions to non-zero | Register x9 = 60 (0x3C). Written by add x9, x8, x3 after MEM→EX forwarding. |
| [8] | Transitions to non-zero | Register x8 = 30 (0x1E). Written by lw x8, 0(x0) after loading from memory. |
| [7]–[5] | Transition then stable | Registers x7=30, x6=30, x5=0. Logic operations (XOR, OR, AND) on x1 and x2. |
| [4] | Transitions to non-zero | Register x4 = −10 (0xFFFFFFF6). Most bits are 1 (two’s complement negative). |
| [3] | Transitions to non-zero | Register x3 = 30. Written by add x3, x1, x2. |
| [2]–[1] | Early transitions | Registers x2=20 and x1=10. First values written after pipeline fills. |
| [0] | Flat at 0 (green) | Register x0 is HARDWIRED to zero in RISC-V. regfile.v ignores writes to x0. |

## 6.3 Signal Color Guide for Vivado Waveforms

Understanding Vivado’s color coding is essential for reading waveforms:

| Color | Meaning | What It Looks Like | Example in This Simulation |
| --- | --- | --- | --- |
| Green | Valid, known logic value (0 or 1) | Sharp, clean transitions between high and low | clk, rst, cycle_cnt, all pipeline signals during normal operation |
| Red | Unknown (X) or uninitialized | Solid colored band, often with hatch marks | i, r_val, dmem_val before the testbench verification phase assigns them |
| Blue | High-impedance (Z) / floating | Usually a thin line at mid-level | Not present in this simulation (no tri-state signals) |
| Yellow line | Time cursor position | Vertical line across all signals | Click anywhere on the waveform to place a cursor and read exact values |

For multi-bit bus signals (like pc_if[31:0] or cycle_cnt[31:0]): Vivado displays the hexadecimal value inside the green waveform band when the value is stable. When the value changes, you see an "X" shaped crossing. The height of the green band does NOT represent the numeric value — it just shows the signal is valid.

# 7. Complete Summary of Expected Results

The following table provides the complete verification matrix for the simulation. Every row must show PASS for the processor to be considered correct:

| # | Check | What It Verifies | Expected | Result | Source File / Line |
| --- | --- | --- | --- | --- | --- |
| 1 | x1 = 10 | I-type ADDI instruction | 10 | PASS | tb_riscv.v line 107 |
| 2 | x2 = 20 | I-type ADDI instruction | 20 | PASS | tb_riscv.v line 108 |
| 3 | x3 = 30 | R-type ADD + EX→EX forwarding | 30 | PASS | tb_riscv.v line 109 |
| 4 | x4 = −10 | R-type SUB + two’s complement | 0xFFFFFFF6 | PASS | tb_riscv.v line 110 |
| 5 | x5 = 0 | R-type AND (bitwise) | 0 | PASS | tb_riscv.v line 111 |
| 6 | x6 = 30 | R-type OR (bitwise) | 30 | PASS | tb_riscv.v line 112 |
| 7 | x7 = 30 | R-type XOR (bitwise) | 30 | PASS | tb_riscv.v line 113 |
| 8 | x8 = 30 | Load Word (LW) + store→load path | 30 | PASS | tb_riscv.v line 114 |
| 9 | x9 = 60 | ADD + MEM→EX forwarding | 60 | PASS | tb_riscv.v line 115 |
| 10 | x10 = 0 | Branch flush (instruction skipped) | 0 | PASS | tb_riscv.v line 116 |
| 11 | x11 = 77 | Branch target execution | 77 | PASS | tb_riscv.v line 117 |
| 12 | dmem[0] = 30 | Store Word (SW) to memory | 30 | PASS | tb_riscv.v line 121 |
| 13 | EX→EX fwd | 1-cycle data forwarding | x3=30 | PASS | tb_riscv.v line 124 |
| 14 | MEM→EX fwd | 2-cycle data forwarding | x9=60 | PASS | tb_riscv.v line 125 |
| 15 | Branch flush | Control hazard handling | x10=0 | PASS | tb_riscv.v line 129 |

Final Verdict If ALL 15 checks show PASS, the 5-stage pipelined RISC-V processor is FULLY FUNCTIONAL. The following features have been verified: • Instruction Fetch, Decode, Execute, Memory Access, and Write-Back stages all work correctly • All RV32I instructions tested (ADD, SUB, AND, OR, XOR, ADDI, LW, SW, BEQ) produce correct results • Data forwarding (EX→EX and MEM→EX) resolves RAW hazards without stalls • Load-use hazard detection correctly stalls the pipeline for 1 cycle • Branch flush correctly discards incorrectly fetched instructions • The register file, data memory, ALU, and all pipeline registers operate correctly

# 8. Glossary of Terms

This glossary defines all technical terms used in this document, arranged alphabetically for quick reference:

| Term | Definition |
| --- | --- |
| ALU | Arithmetic Logic Unit. The hardware block that performs mathematical operations (add, subtract) and logic operations (AND, OR, XOR). In our processor: alu.v. |
| Branch | A conditional jump in the program. The processor checks a condition (e.g., are two registers equal?) and either continues to the next instruction or jumps to a different address. Example: BEQ (Branch if Equal). |
| Clock (clk) | A signal that oscillates between 0 and 1 at a fixed frequency. All synchronous operations in the processor happen on the rising edge (0→1 transition) of the clock. Our clock runs at 100 MHz (10 ns period). |
| Control Hazard | A pipeline problem caused by branch instructions. The processor doesn’t know where to go next until the branch is resolved, but it has already fetched the next instructions. Solved by flushing. |
| Data Hazard (RAW) | Read After Write. A pipeline problem where an instruction needs a value that a previous instruction hasn’t finished writing yet. Solved by forwarding or stalling. |
| DUT | Device Under Test. The main processor module (riscv_top) being tested by the testbench. |
| Flush | The act of clearing pipeline registers to discard incorrectly fetched instructions. Used when a branch is taken or a jump executes. Replaces instructions with NOPs (no operations). |
| Forwarding | A technique to resolve data hazards by routing a value directly from where it is available (a pipeline register) to where it is needed (the ALU input), bypassing the register file. Also called bypassing. |
| Hazard | Any situation that prevents the next instruction from executing in the next clock cycle. Three types: data hazards, control hazards, and structural hazards (not present in our design). |
| Immediate (imm) | A constant value embedded directly within the instruction encoding. For example, in addi x1, x0, 10, the value 10 is the immediate. Extracted and sign-extended by imm_gen.v. |
| Load-Use Hazard | A special case of data hazard: a load instruction (LW) is immediately followed by an instruction that uses the loaded value. Cannot be solved by forwarding alone — requires a 1-cycle stall. |
| Little-Endian | A byte ordering where the least significant byte is stored at the lowest memory address. RISC-V uses little-endian. Example: the 32-bit value 30 (0x0000001E) is stored as bytes: 1E, 00, 00, 00. |
| NOP | No Operation. An instruction that does nothing. Encoded as addi x0, x0, 0 (hex: 0x00000013). Used to fill pipeline bubbles after stalls or flushes. |
| Opcode | The 7-bit field in a RISC-V instruction (bits [6:0]) that identifies the instruction type. For example: 0110011 = R-type, 0010011 = I-type ALU, 1100011 = Branch. |
| PC (Program Counter) | A register that holds the address of the current instruction being fetched. It normally increments by 4 each cycle but can be redirected by branches or jumps. |
| Pipeline Register | A register that sits between two pipeline stages and stores all data and control signals as they pass from one stage to the next. Updated on every rising clock edge. |
| Register File | A small, fast memory inside the processor that holds 32 registers (x0–x31), each 32 bits wide. x0 is hardwired to 0. Instructions read from and write to these registers. In our processor: regfile.v. |
| Reset (rst) | A signal that initializes all hardware to a known state. When rst=1, all registers, pipeline latches, and the PC are cleared to 0. The processor begins execution when rst goes to 0. |
| RV32I | The base integer instruction set of RISC-V, supporting 32-bit operations. The “I” stands for “Integer.” Our processor implements a subset of RV32I. |
| Stall | Pausing the pipeline for one or more cycles. During a stall, the PC and IF/ID register hold their values, and a NOP bubble is inserted into the pipeline. Used for load-use hazards. |
| Testbench | A Verilog module (tb_riscv.v) that is NOT part of the actual hardware. It generates clock and reset signals, runs the simulation, and checks results using $display and direct access to internal signals. |
| Two’s Complement | The standard way to represent negative numbers in binary. To negate a number: invert all bits and add 1. Example: −10 = ~10 + 1 = ~0x0000000A + 1 = 0xFFFFFFF5 + 1 = 0xFFFFFFF6. |
| WB (Write-Back) | The final pipeline stage where the computed result is written back into the register file. |
| Waveform | A graphical representation of a signal’s value over time. In Vivado’s waveform viewer, 1-bit signals appear as square waves and multi-bit signals appear as bands with hex values. |

# 9. Troubleshooting Common Issues

If you encounter problems, use this guide to diagnose and fix them:

| Problem | Possible Cause | How to Fix |
| --- | --- | --- |
| Simulation does not start | tb_riscv is not set as the top simulation module | Right-click SIMULATION in Flow Navigator → Simulation Settings → set top module to tb_riscv |
| Waveforms are empty or flat | Simulation not run; still paused at time 0 | Click Run All button (solid triangle) or press Shift+F5 to run until $finish |
| All signals show X (red) | Reset not working; pipeline registers not initialized | Check that rst is connected to all pipeline registers. Ensure rst stays high for at least 2 clock cycles. |
| Some registers show FAIL | Bug in the corresponding pipeline stage | Identify which register failed. Trace the instruction backward through EX, ID, IF stages. Check control signals, ALU operation, and forwarding. |
| x10 shows FAIL (value ≠ 0) | Branch flush not working | Check flush_pipe signal in riscv_top.v. Verify if_id_reg.v and id_ex_reg.v clear on flush. Check branch_taken logic. |
| Forwarding test fails | Forwarding unit not detecting dependency | Check forward_a / forward_b in waveform. Verify conditions in forward_unit.v. Check that ex_mem_regwrite and mem_wb_regwrite are correctly propagated. |
| Simulation times out (>2000 ns) | Halt instruction not working | Check BEQ encoding at mem[14] in instr_mem.v (should be 0x00000063). Verify branch logic handles beq x0, x0, 0. |
| Register values appear one cycle late | This is normal pipeline behavior | Register writes occur in WB stage (5th stage). A 5-cycle delay from fetch to write-back is expected. |
| pc_if shows unexpected values | Possible stall or flush issue | Check stall and flush_pipe signals in the waveform. Compare pc_if values with expected instruction addresses. |
| Waveform config file missing | tb_riscv_behav.wcfg not found | Go to File → Open Simulation Waveform Configuration. Navigate to the project directory and select the .wcfg file. |