LINE-BY-LINE CODE EXPLANATION

Every Verilog File in the RISC-V 5-Stage Pipeline

with Hardware Taint Tracking (DIFT)

Written in simple English for team understanding

# File 1: pc_reg.v (Program Counter)

PURPOSE: This module holds the address of the instruction currently being fetched. It updates every clock cycle to point to the next instruction.

module pc_reg (

This line starts the definition of a new hardware module called 'pc_reg'. A module is like a building block — it has inputs, outputs, and internal logic.

input  wire        clk,

This declares an input called 'clk' (clock). It is 1 bit wide. The clock is a square wave signal that toggles between 0 and 1. Everything in the processor is synchronized to this clock.

input  wire        rst,

This declares an input called 'rst' (reset). When rst=1, the processor goes back to its starting state. When rst=0, it runs normally.

input  wire        stall,

This declares an input called 'stall'. When stall=1, the PC must FREEZE — it keeps its current value and does NOT update. This is used during load-use hazards.

input  wire [31:0] pc_next,

This declares a 32-bit input called 'pc_next'. This is the NEXT address the PC should jump to. It could be PC+4 (next instruction) or a branch target address.

output reg  [31:0] pc

This declares a 32-bit output called 'pc'. The keyword 'reg' means it holds its value between clock edges (it is a flip-flop, not just a wire). This is the CURRENT program counter value.

);

This closes the port list of the module.

always @(posedge clk or posedge rst) begin

This is a 'sequential logic block'. It runs every time the clock goes from 0 to 1 (posedge clk = positive edge of clock) OR when reset goes from 0 to 1 (posedge rst). This is how we create flip-flops in Verilog.

if (rst)

Check if reset is active (rst=1). If yes, we must set the PC to its initial value.

pc <= 32'h0000_0000;

Set PC to address 0 (the very first instruction in memory). The '<=' symbol is a 'non-blocking assignment' — it means the value updates at the END of the clock edge, not immediately. 32'h means a 32-bit hexadecimal number.

else if (!stall)

If reset is NOT active, check if stall is NOT active (!stall means stall=0). Only update PC if we are NOT stalled.

pc <= pc_next;

Update the PC to the next address. If stall=1, this line is SKIPPED — the PC keeps its old value (frozen). This is how load-use hazards pause the pipeline.

end

Closes the always block.

endmodule

Closes the module definition. The pc_reg module is complete.

# File 2: instr_mem.v (Instruction Memory)

PURPOSE: This is a read-only memory that stores the program. Given an address (the PC), it outputs the 32-bit instruction at that address.

module instr_mem (

Start of the instruction memory module.

input  wire [31:0] addr,

Input: the byte address to read from. This comes from the Program Counter. It is 32 bits wide, but only the lower bits are actually used (since memory is small).

output wire [31:0] instr

Output: the 32-bit instruction stored at that address.

reg [31:0] mem [0:255];

Create an array of 256 words, each 32 bits wide. This is the instruction memory — it can hold up to 256 instructions (1 KB total). 'mem[0]' is the first instruction, 'mem[1]' is the second, etc.

integer i;

Declare an integer variable 'i' for use in loops. This is only used during simulation initialization, not in real hardware.

initial begin

'initial begin' runs ONCE at the start of simulation (time 0). It is used to pre-load the memory with our test program. This does NOT synthesize to hardware — it is only for simulation.

for (i = 0; i < 256; i = i + 1)

Loop through all 256 memory locations.

mem[i] = 32'h0000_0013;

Fill every location with a NOP instruction (No Operation). The hex value 0x00000013 is the encoding of 'addi x0, x0, 0' which does nothing. This ensures unused memory locations don't cause errors.

mem[0]  = 32'h00A00093;   // addi x1, x0, 10

Store instruction 'addi x1, x0, 10' at address 0. This means: x1 = x0 + 10 = 0 + 10 = 10. The hex value 0x00A00093 is the binary encoding of this instruction.

mem[7]  = 32'h00302023;   // sw x3, 0(x0)

Store instruction 'sw x3, 0(x0)' at address 7. This means: store the value of x3 into data memory at address (x0 + 0) = address 0. SW = Store Word.

mem[8]  = 32'h00002403;   // lw x8, 0(x0)

Load instruction 'lw x8, 0(x0)'. This means: load 32 bits from data memory address 0 into register x8. LW = Load Word. This creates a load-use hazard if the next instruction uses x8.

mem[11] = 32'h00318463;   // beq x3, x3, +8

Branch instruction 'beq x3, x3, +8'. This means: if x3 equals x3 (always true!), jump forward by 8 bytes. This tests branch flushing — the pipeline must throw away the wrongly-fetched instruction after this.

assign instr = mem[addr[9:2]];

This is the READ logic. addr[9:2] takes bits 9 down to 2 of the address (throwing away the bottom 2 bits). Why? Because each instruction is 4 bytes, so addresses go 0, 4, 8, 12... Dividing by 4 (which is the same as removing the bottom 2 bits) gives the word index: 0, 1, 2, 3...

endmodule

End of instruction memory module.

# File 3: control_unit.v (Main Decoder)

PURPOSE: The 'brain' of the processor. It reads the 7-bit opcode from the instruction and generates ALL the control signals that tell every other module what to do.

input  wire [6:0] opcode,

Input: the 7-bit opcode from bits [6:0] of the instruction. This tells us what TYPE of instruction it is (R-type, I-type, load, store, branch, etc.).

output reg regwrite,

Output: regwrite=1 means 'write the result to a register'. For ADD, ADDI, LW, JAL: regwrite=1. For SW, BEQ: regwrite=0 (they don't write to a register).

output reg memread,

Output: memread=1 means 'read from data memory'. Only LW (Load Word) sets this to 1.

output reg memwrite,

Output: memwrite=1 means 'write to data memory'. Only SW (Store Word) sets this to 1.

output reg alusrc,

Output: alusrc controls what goes into the ALU's second input. 0 = use the register value (R-type: add x3, x1, x2). 1 = use the immediate value (I-type: addi x1, x0, 5).

output reg [1:0] wb_sel,

Output: wb_sel selects what data gets written back to the register file. 00 = ALU result. 01 = data from memory (LW). 10 = PC+4 (for JAL/JALR to save the return address).

output reg [1:0] aluop,

Output: aluop tells the ALU decoder what category of operation this is. 00 = ADD (for loads/stores). 01 = branch. 10 = R-type or I-type ALU (look at funct3/funct7 for details). 11 = LUI (pass immediate through).

localparam OP_R = 7'b0110011;

Define a constant: the opcode for R-type instructions (ADD, SUB, AND, OR, XOR, etc.) is binary 0110011 = decimal 51. 'localparam' creates a named constant that makes the code readable.

always @(*) begin

'always @(*)' is combinational logic — it runs whenever ANY input changes (no clock needed). This is a pure decoder: input opcode in, control signals out, instantly.

regwrite=0; memread=0; memwrite=0; ...

Set ALL outputs to safe defaults (all zeros = NOP). This is important — if the opcode doesn't match any known instruction, all signals stay at 0 and nothing harmful happens.

case (opcode)

Start a case statement. This checks the opcode value and executes the matching block.

OP_R: begin regwrite=1; aluop=2'b10; end

If opcode = R-type (0110011): enable register write (we're writing a result), set aluop=10 (tell ALU decoder to look at funct3/funct7 for the specific operation like ADD vs SUB).

OP_I_ALU: begin regwrite=1; alusrc=1; aluop=2'b10; end

If opcode = I-type ALU (0010011, e.g., ADDI): enable register write, set alusrc=1 (use immediate as second ALU input instead of register), aluop=10.

OP_LOAD: begin regwrite=1; memread=1; alusrc=1; wb_sel=2'b01; end

If opcode = Load (0000011, e.g., LW): enable register write, enable memory read, use immediate (for address calculation), wb_sel=01 (write memory data to register, not ALU result).

OP_STORE: begin memwrite=1; alusrc=1; end

If opcode = Store (0100011, e.g., SW): enable memory write, use immediate (for address calculation). Note: regwrite stays 0 — stores don't write to registers.

OP_BRANCH: begin branch=1; aluop=2'b01; end

If opcode = Branch (1100011, e.g., BEQ): set branch=1 (tells the pipeline this is a branch). No regwrite — branches don't write to registers.

# File 4: alu.v (Arithmetic Logic Unit)

PURPOSE: The ALU does ALL the math in the processor. It takes two 32-bit numbers and a 4-bit control signal, and produces a 32-bit result.

input  wire [31:0] a, b,

Two 32-bit inputs: 'a' is the first operand (usually from rs1), 'b' is the second operand (either from rs2 or the immediate value, depending on alusrc).

input  wire [3:0]  alu_ctrl,

4-bit control input from the ALU decoder. This tells the ALU WHICH operation to perform (add, subtract, AND, OR, etc.).

output reg  [31:0] result,

32-bit output: the result of the operation.

assign zero = (result == 32'b0);

The 'zero' flag is 1 when the result equals zero. Used by BEQ: if a-b=0, then a equals b.

4'b0000: result = a + b;      // ADD

When alu_ctrl = 0000: add the two inputs. Used for ADD, ADDI, and address calculation for LW/SW.

4'b0001: result = a - b;      // SUB

When alu_ctrl = 0001: subtract b from a. Used for SUB instruction.

4'b0010: result = a & b;      // AND

When alu_ctrl = 0010: bitwise AND. Each bit of the result is 1 only if BOTH corresponding bits of a and b are 1.

4'b0011: result = a | b;      // OR

When alu_ctrl = 0011: bitwise OR. Each bit of the result is 1 if EITHER bit of a or b is 1.

4'b0100: result = a ^ b;      // XOR

When alu_ctrl = 0100: bitwise XOR (exclusive OR). Each bit is 1 if the two corresponding bits are DIFFERENT.

4'b0101: result = a << b[4:0];  // SLL

When alu_ctrl = 0101: shift left logical. Shift 'a' left by b[4:0] positions (only bottom 5 bits of b are used, since max shift for 32-bit is 31). Zeros fill from the right.

4'b0111: result = $signed(a) >>> b[4:0]; // SRA

When alu_ctrl = 0111: shift right arithmetic. Shift 'a' right, but fill from the left with the SIGN BIT (bit 31). This preserves the sign of negative numbers.

4'b1000: result = ($signed(a) < $signed(b)) ? 1 : 0; // SLT

When alu_ctrl = 1000: Set Less Than (signed). If a < b (treating both as signed numbers), result = 1. Otherwise result = 0.

4'b1010: result = b;            // PASS-B (LUI)

When alu_ctrl = 1010: just pass input 'b' through to the output. Used for LUI (Load Upper Immediate): the immediate value goes directly to the result.

# File 5: alu_ctrl.v (ALU Control Decoder)

PURPOSE: Translates the 2-bit aluop (from control unit) + funct3 + funct7 into the 4-bit alu_ctrl signal that tells the ALU exactly which operation to do.

2'b00: alu_ctrl = 4'b0000;  // ADD

When aluop=00 (load, store, AUIPC, JAL): always ADD. Loads/stores need addition to calculate the memory address (base + offset).

2'b11: alu_ctrl = 4'b1010;  // PASS-B (LUI)

When aluop=11 (LUI): pass the immediate value through the ALU unchanged.

2'b10: begin ... end  // R/I-type

When aluop=10 (R-type or I-type ALU): we need to look at funct3 (and sometimes funct7) to determine the exact operation.

3'b000: alu_ctrl = (funct7[5]) ? 4'b0001 : 4'b0000;

When funct3=000: this could be ADD or SUB. We check funct7 bit 5: if it's 1, it's SUB (0001). If it's 0, it's ADD (0000). This is how RISC-V distinguishes ADD from SUB — they have the same funct3 but different funct7.

# File 6: imm_gen.v (Immediate Generator)

PURPOSE: Different instruction formats store the immediate (constant number) in different bit positions. This module extracts and sign-extends the immediate to 32 bits.

OP_I_ALU, OP_LOAD, OP_JALR:

For I-type instructions (ADDI, LW, JALR): the immediate is in bits [31:20] of the instruction.

imm = {{20{instr[31]}}, instr[31:20]};

Sign-extend the 12-bit immediate to 32 bits. '{20{instr[31]}}' means: repeat bit 31 (the sign bit) twenty times. If the number is negative (bit 31=1), the top 20 bits are all 1s. If positive (bit 31=0), the top 20 bits are all 0s. Then concatenate with the 12-bit immediate.

OP_STORE:

For S-type instructions (SW): the immediate is split across two fields: bits [31:25] and bits [11:7].

imm = {{20{instr[31]}}, instr[31:25], instr[11:7]};

Concatenate the two fields and sign-extend. The immediate = {instr[31:25], instr[11:7]} = 12 bits, then sign-extended to 32 bits.

OP_BRANCH:

For B-type instructions (BEQ, BNE): the immediate bits are scattered across the instruction in a specific pattern.

imm = {{19{instr[31]}}, instr[31], instr[7], instr[30:25], instr[11:8], 1'b0};

Reconstruct the 13-bit branch offset from scattered bits and sign-extend. The last bit is always 0 (branches are always to even addresses, aligned to 2 bytes).

OP_LUI, OP_AUIPC:

For U-type instructions: the immediate is in bits [31:12] (the upper 20 bits).

imm = {instr[31:12], 12'b0};

Place the 20-bit immediate into the upper 20 bits, and fill the lower 12 bits with zeros. This gives a value aligned to 4096.

# File 7: data_mem.v (Data Memory)

PURPOSE: 1 KB byte-addressable data memory for load (LW) and store (SW) instructions. Supports word, half-word, and byte access.

reg [7:0] mem [0:1023];

Create an array of 1024 bytes (1 KB). Each element is 8 bits (1 byte). This is byte-addressable memory — each byte has its own address.

always @(posedge clk) begin  // Synchronous Write

Writes happen on the rising edge of the clock. This ensures data is stored at a predictable time.

if (memwrite) begin

Only write if the memwrite control signal is active (SW instruction).

3'b010: {mem[addr+3],mem[addr+2],mem[addr+1],mem[addr]} <= wdata;

SW (Store Word): write all 4 bytes. The {} concatenation operator packs 4 bytes into 32 bits. Little-endian: the least significant byte goes to the lowest address.

always @(*) begin  // Combinational Read

Reads happen instantly (combinational, no clock). When memread=1 and the address changes, the output updates immediately.

3'b010: rdata = {mem[addr+3],mem[addr+2],mem[addr+1],mem[addr]};

LW (Load Word): read 4 consecutive bytes and combine them into a 32-bit word.

3'b000: rdata = {{24{mem[addr][7]}}, mem[addr]};

LB (Load Byte, sign-extended): read 1 byte and sign-extend to 32 bits. mem[addr][7] is the sign bit of that byte.

# File 8: forward_unit.v (Data Forwarding Unit)

PURPOSE: Detects data hazards (when an instruction needs a value still in the pipeline) and selects the correct forwarded value, avoiding stalls.

output reg [1:0] forward_a, forward_b

2-bit outputs: forward_a controls the first ALU input (rs1), forward_b controls the second (rs2). 00 = use register file (no hazard). 10 = forward from EX/MEM. 01 = forward from MEM/WB.

if (ex_mem_regwrite && (ex_mem_rd != 5'b0) && (ex_mem_rd == id_ex_rs1))

Check THREE conditions: (1) The instruction in EX/MEM is writing to a register. (2) The destination is NOT x0 (x0 is always 0, never forwarded). (3) The destination MATCHES the source register rs1 of the current EX instruction. If ALL three are true, we have a hazard!

forward_a = 2'b10;   // EX/MEM -> EX (highest priority)

Forward the ALU result from the PREVIOUS instruction (1 cycle ago). This is EX-to-EX forwarding — the most recent result gets priority.

else if (mem_wb_regwrite && (mem_wb_rd != 5'b0) && (mem_wb_rd == id_ex_rs1))

If no EX/MEM hazard, check MEM/WB (2 cycles ago). Same three conditions but checking the MEM/WB pipeline register.

forward_a = 2'b01;   // MEM/WB -> EX

Forward the write-back data from 2 instructions ago.

else forward_a = 2'b00;  // register file (no hazard)

No hazard detected — use the normal register file value.

# File 9: hazard_unit.v (Load-Use Hazard Detection)

PURPOSE: Detects when a load instruction (LW) is immediately followed by an instruction that needs the loaded value. Inserts a 1-cycle stall because forwarding alone cannot fix this.

assign stall = id_ex_memread &&

First condition: the instruction currently in ID/EX is a LOAD (memread=1). If it's not a load, no stall is needed.

((id_ex_rd == if_id_rs1) || (id_ex_rd == if_id_rs2)) &&

Second condition: the load's destination register matches EITHER source register of the instruction in IF/ID. This means the next instruction NEEDS the loaded value.

(id_ex_rd != 5'b0);

Third condition: the destination is not x0 (x0 is always 0, loading into x0 is meaningless). If all three conditions are true, stall=1 — freeze the pipeline for 1 cycle.

When stall=1: PC freezes, IF/ID holds, ID/EX gets a bubble (NOP). After 1 cycle, the load data is in MEM/WB and can be forwarded normally.

# File 10: if_id_reg.v (IF/ID Pipeline Register)

PURPOSE: Stores the PC value and instruction word between the IF and ID stages. Supports flush (on branch/jump) and stall (on load-use hazard).

input  wire flush,   // insert NOP on branch/jump

When flush=1, the instruction in this register is WRONG (fetched before we knew the branch was taken). We must replace it with a NOP.

input  wire stall,   // hold current content

When stall=1, we must keep the current instruction (do not update). This gives the pipeline time to resolve a load-use hazard.

if (rst || flush) begin

On reset OR flush: clear the register to a NOP.

instr_out <= 32'h0000_0013; // NOP

Insert a NOP (addi x0, x0, 0). This 'bubble' harmlessly flows through the pipeline without doing anything.

end else if (!stall) begin

If not flushed and not stalled: normal operation — latch the new PC and instruction from the IF stage.

// when stall=1 and no flush: hold values (do nothing)

If stall=1 (and no flush), the register keeps its current values. The same instruction will be decoded AGAIN next cycle.

# File 11: id_ex_reg.v (ID/EX Pipeline Register)

PURPOSE: Carries ALL data and control signals from the ID stage to the EX stage. This is the largest pipeline register — it carries the PC, register data, immediate, register addresses, all control signals, and taint bits.

input  wire [31:0] rdata1_in, rdata2_in, imm_in,

Data inputs: rdata1 = value of source register rs1, rdata2 = value of source register rs2, imm = the sign-extended immediate value.

input  wire [4:0]  rs1_in, rs2_in, rd_in,

Register addresses: rs1 = source register 1 number (0-31), rs2 = source register 2 number, rd = destination register number.

input  wire        taint1_in, taint2_in,

TAINT INPUTS (novel addition): taint1 = taint bit of rs1, taint2 = taint bit of rs2. These travel alongside the data through the pipeline.

if (rst || flush) begin ... all outputs <= 0 ... end

On reset or flush: clear everything to zero. This inserts a NOP bubble — all control signals become 0, so nothing happens in the EX stage.

else begin ... all outputs <= inputs ... end

Normal operation: copy all inputs to outputs on the clock edge. The data moves from ID to EX in one cycle.

# File 12-13: ex_mem_reg.v and mem_wb_reg.v

PURPOSE: Pipeline registers between EX/MEM and MEM/WB stages. Same concept as id_ex_reg but carrying fewer signals (only what later stages need).

## ex_mem_reg.v (EX to MEM)

input wire taint_rd_in,    // taint of ALU result

TAINT: carries the propagated taint of the destination register (taint_a OR taint_b). This will be forwarded to the next instruction if there is a data hazard.

input wire taint_rs2_in,   // taint of store data

TAINT: carries the taint of rs2 (the data being stored in SW instructions). The taint_unit checks this in the MEM stage to detect tainted stores.

## mem_wb_reg.v (MEM to WB)

input wire taint_wb_in,    // taint arriving at WB

TAINT: carries the taint that will be written back to the register file alongside the data. If this instruction was tainted, the destination register will be marked tainted.

# File 14: taint_regfile.v (Taint-Aware Register File) — NEW

PURPOSE: This REPLACES the original regfile.v. It adds a 32-bit 'taints' register where each bit tracks whether the corresponding register is tainted. This is the CORE of our novel contribution.

reg [31:0] regs  [1:31];

The normal register data: 31 registers (x1 to x31), each 32 bits. x0 is hardwired to 0 and not stored.

reg [31:0] taints;

THE KEY ADDITION: a single 32-bit register where each bit is the taint tag for the corresponding register. taints[1] = taint of x1, taints[5] = taint of x5, etc.

input  wire        wtaint,

Input: the taint bit to write to the destination register during write-back. This comes from the pipeline (taint_wb after the WB mux).

input  wire [31:0] taint_src_mask,

Input: a 32-bit mask from the CSR that defines which registers START as tainted. Bit i=1 means register xi is a taint source.

input  wire        taint_init,

Input: a 1-cycle pulse that tells the register file to load the taint_src_mask into the taints register. This initializes the taint state.

if (rst) taints <= 32'b0;

On reset: clear all taint bits. All registers start clean.

else if (taint_init) taints <= taint_src_mask;

On taint_init pulse: load the taint source mask from CSR. Example: if taint_src_mask = 0x00000020 (bit 5 = 1), then taints[5] = 1 (x5 is now tainted).

else if (regwrite && rd != 5'b0) begin

On normal write-back: if we're writing to a register (and it's not x0)...

regs[rd]   <= wdata;

Write the 32-bit data value to the register.

taints[rd] <= wtaint;

Write the 1-bit taint tag to the corresponding bit in taints. If the computation was tainted, this register is now tainted.

assign taint1 = (rs1 == 5'b0) ? 1'b0 :

Read taint of rs1: if rs1 is x0, taint is always 0 (x0 is never tainted). Otherwise, check write-through first (same as data read), then read from taints register.

assign taint_status = {taints[31:1], 1'b0};

Output the full 32-bit taint state for the CSR_TAINT_STATUS register. Bit 0 is forced to 0 (x0 is never tainted).

# File 15: taint_unit.v (Taint Propagation + Violation Detection) — NEW

PURPOSE: This is the HEART of the security system. It does two things: (1) propagates taint through computation, and (2) detects when tainted data tries to reach protected memory.

input  wire taint_a,

Input: the taint of ALU operand A (rs1 after forwarding). This comes from the taint forwarding mux in riscv_top.v.

input  wire taint_b,

Input: the taint of ALU operand B (rs2 or immediate, after forwarding).

assign taint_result = taint_a | taint_b;

TAINT PROPAGATION: the result taint = taint_a OR taint_b. This ONE line implements the core security rule: if EITHER input is tainted, the output is tainted. This is the information flow tracking rule.

input  wire taint_en,

Input: global taint enable from CSR. When 0, no violations are reported (tracking disabled).

input  wire memwrite,

Input: is this a store instruction in the MEM stage?

input  wire taint_store_data,

Input: is the DATA being stored tainted? This is taint_rs2_mem from the EX/MEM register.

input  wire [31:0] store_addr, prot_base, prot_bound,

Inputs: the store address (ALU result from EX/MEM) and the protected memory region boundaries (from CSR).

assign addr_in_protected = (store_addr >= prot_base) && (store_addr < prot_bound);

Check if the store address falls within the protected region. This is a simple range check: base <= addr < bound.

assign taint_violation = taint_en && memwrite && taint_store_data && addr_in_protected;

VIOLATION DETECTION: ALL FOUR conditions must be true. (1) Tracking enabled. (2) It's a store. (3) Data is tainted. (4) Address is protected. If even ONE is false, no violation — this is why we get zero false positives.

# File 16: csr_taint.v (Custom CSR Registers) — NEW

PURPOSE: Six custom Control and Status Registers that let software configure the taint tracking policy: what to taint, what to protect, and whether tracking is enabled.

localparam CSR_TAINT_EN     = 12'h800;

CSR address 0x800: the taint enable register. Writing 1 to this turns on taint tracking.

localparam CSR_TAINT_SRC    = 12'h801;

CSR address 0x801: the taint source mask. Each bit corresponds to a register. Bit 5 = 1 means x5 is tainted.

localparam CSR_PROT_BASE    = 12'h803;

CSR address 0x803: the base address of the protected memory region.

localparam CSR_PROT_BOUND   = 12'h804;

CSR address 0x804: the bound (end) address of the protected memory region.

if (taint_violation) excpt_count <= excpt_count + 1;

Every time a taint violation occurs, increment the exception counter. This tracks how many attacks have been caught.

CSR_TAINT_SRC: begin taint_src_mask <= csr_wdata; taint_init <= 1'b1; end

When writing to CSR 0x801: store the new mask AND automatically pulse taint_init for 1 cycle. This triggers the register file to load the new taint configuration immediately.

always @(*) begin case (csr_addr) ... endcase end

CSR read logic: combinational multiplexer that outputs the value of whichever CSR is being read, based on the address.

# File 17: riscv_top.v (Top-Level Module) — MODIFIED

PURPOSE: This is the TOP module that WIRES everything together. Every other module is instantiated here and connected through wires. The taint signals flow alongside data through the entire pipeline.

## Key Taint Wiring in riscv_top.v

wire taint1_id, taint2_id;

Taint bits read from the register file in the ID stage. taint1_id = taint of rs1, taint2_id = taint of rs2.

wire taint1_ex, taint2_ex;

Taint bits arriving in the EX stage from the ID/EX pipeline register.

wire taint_a, taint_b;

FORWARDED taint bits. These are the ACTUAL taint values used by the taint_unit — they account for data forwarding.

## Taint Forwarding Muxes (The Critical Part)

assign taint_a = (forward_a == 2'b10) ? taint_rd_mem :

If data forwarding selects EX/MEM (1-cycle-ago result), also forward its taint.

(forward_a == 2'b01) ? taint_wb_wb  :

If data forwarding selects MEM/WB (2-cycles-ago result), also forward its taint.

taint1_ex;

If no forwarding needed, use the taint from the ID/EX register (normal read from register file).

These taint forwarding muxes MIRROR the data forwarding muxes exactly. This is CRITICAL — without this, a forwarded value would have the wrong taint bit, and the security system would miss attacks or produce false positives.

## WB Taint Mux

assign wtaint = (wb_sel_wb == 2'b10) ? 1'b0 : taint_wb_wb;

The write-back taint mux. If wb_sel=10 (writing PC+4 from JAL/JALR), the taint is ALWAYS 0 — the program counter is trusted. Otherwise, use the taint from the pipeline (ALU result taint or load address taint).

## Taint Exception Output

assign taint_exception = taint_violation_mem;

Expose the taint violation signal as a top-level output. This can be used to halt the pipeline, trigger an interrupt, or (in our demo) display 'ATTACK DETECTED!' in the testbench.

# File 18: tb_riscv.v (Original Pipeline Testbench)

PURPOSE: This testbench verifies the ORIGINAL 5-stage pipeline works correctly. It runs the built-in test program (15 instructions) and checks all register values. Taint tracking is DISABLED (csr_we=0).

`timescale 1ns / 1ps

Sets the time unit to 1 nanosecond and precision to 1 picosecond. All delay values (#5, #1) are in nanoseconds.

module tb_riscv;

Start of the testbench module. Testbenches have NO inputs or outputs — they generate all signals internally.

reg  clk;

Declare a register for the clock. We use 'reg' (not 'wire') because the testbench DRIVES this signal.

reg  rst;

Declare a register for reset. The testbench controls when reset is active.

wire csr_we    = 1'b0;

Taint CSR write enable is TIED TO ZERO. This means taint tracking is completely disabled for this test. The original pipeline functionality is tested in isolation.

wire [11:0] csr_addr  = 12'h0;

CSR address tied to 0 (not used).

wire [31:0] csr_wdata = 32'h0;

CSR write data tied to 0 (not used).

riscv_top DUT ( .clk(clk), .rst(rst), ... );

Instantiate the Device Under Test (DUT) — our complete processor. Connect clock, reset, and all taint ports. The taint ports are tied off to disabled values.

initial clk = 0;

Initialize clock to 0 at time 0.

always #5 clk = ~clk;

Toggle the clock every 5ns. This creates a 10ns period (100 MHz) square wave: 0 for 5ns, 1 for 5ns, 0 for 5ns, etc.

task read_reg; input [4:0] reg_num; output [31:0] val; begin

A 'task' is a reusable block of code (like a function). This task reads a register value directly from the register file using hierarchical access (DUT.RF.regs[reg_num]).

if (reg_num == 0) val = 32'b0;

x0 is always 0 (not stored in the array), so return 0 directly.

else val = DUT.RF.regs[reg_num];

For x1-x31, read directly from the register file array inside the DUT. 'DUT.RF.regs[5]' means: go inside the DUT, find the RF (register file) module, and read regs[5]. This only works in simulation.

rst = 1; @(posedge clk); #1; @(posedge clk); #1; rst = 0;

Apply reset for 2 clock cycles. rst=1 clears all pipeline registers, PC goes to 0. Then release reset (rst=0) and the pipeline starts executing.

repeat(60) @(posedge clk);

Wait for 60 clock cycles. The 15-instruction program finishes well within this. 'repeat(60)' means: do the following 60 times. '@(posedge clk)' means: wait for one rising clock edge.

read_reg(1, r_val); $display("x1 = %0d (expected 10) %s", ...

Read register x1 and display its value. Check if it matches the expected value (10). If yes, print PASS. If no, print FAIL. This is how we verify the processor works correctly.

$finish;

End the simulation. Without this, the simulation would run forever (clock keeps toggling).

# File 19: tb_riscv_taint.v (Taint Attack Demo Testbench)

PURPOSE: This is the EXPO DEMO testbench. It tells a story in 4 acts: configure security policy, run a program with attacker data, watch taint spread through computation, and catch the attack when tainted data tries to reach protected memory.

## Clock and Reset Setup

reg clk = 0;

Declare clock and initialize to 0 at time 0.

reg rst = 1;

Declare reset and initialize to 1 (active) at time 0. The processor starts in reset.

always #5 clk = ~clk;

Toggle clock every 5ns = 10ns period = 100 MHz. This runs continuously throughout the simulation.

## CSR Interface

reg         csr_we    = 0;

CSR write enable. When we want to configure taint tracking, we set this to 1. Default is 0 (no write).

reg  [11:0] csr_addr  = 12'h0;

CSR address register. We write different CSR addresses (0x800-0x804) to configure different settings.

reg  [31:0] csr_wdata = 32'h0;

CSR write data. The value to write to the selected CSR register.

## DUT Instantiation

riscv_top DUT ( .clk(clk), .rst(rst), .csr_we(csr_we), ... );

Instantiate the processor (DUT = Device Under Test). Connect clock, reset, CSR interface, and taint status outputs. Unlike tb_riscv.v, here the CSR ports are connected to our reg signals so we can actively write to them.

## Monitoring

integer exception_count = 0;

Counter that tracks how many taint violations have been detected. Starts at 0.

integer cycle = 0;

Cycle counter for display purposes.

always @(posedge clk) cycle = cycle + 1;

Increment cycle counter on every rising clock edge.

always @(posedge clk) begin

Exception monitor: runs every clock cycle.

if (taint_exception) exception_count = exception_count + 1;

If the taint_exception signal is HIGH this cycle, increment the exception counter. This catches every violation.

## CSR Write Task

task write_csr; input [11:0] addr; input [31:0] data;

A reusable task that writes a value to a CSR register. Takes the CSR address and data as inputs.

@(negedge clk);

Wait for the falling edge of the clock. We change signals on the falling edge so they are stable when the rising edge arrives (setup time).

csr_we = 1; csr_addr = addr; csr_wdata = data;

Set the CSR write enable to 1, set the address and data. On the NEXT rising clock edge, the csr_taint module will latch these values.

@(negedge clk);

Wait for the next falling edge (one full clock cycle has passed).

csr_we = 0;

Deassert write enable. The CSR write is complete. The value has been stored in the CSR register.

## Show Register Task

task show_reg; input [4:0] rnum; input [255:0] label;

A task that displays a register's value AND its taint status. The label is a string describing what the register holds.

val = DUT.RF.regs[rnum];

Read the register value by reaching inside the DUT hierarchy.

tainted = taint_status[rnum];

Read the taint bit for this register from the taint_status output. taint_status[5] = 1 means x5 is tainted.

if (tainted) $display("TAINTED!");

If the register is tainted, display 'TAINTED!' in red. If clean, display 'clean'.

## Program Loading

DUT.IMEM.mem[0]  = 32'h00000013;  // nop

Load NOP into instruction memory slot 0. We pad 12 NOPs at the start to give the testbench time to configure CSRs BEFORE the real program executes. Without these NOPs, the program would run with taint disabled!

DUT.IMEM.mem[12] = 32'h00500093;  // addi x1, x0, 5

The REAL program starts at slot 12. By the time the pipeline reaches this instruction (~cycle 16), all CSR writes are done and taint tracking is fully configured.

DUT.IMEM.mem[15] = 32'h02A00293;  // addi x5, x0, 42

Load 42 into x5. This simulates attacker-controlled input. x5 is already marked as tainted by the CSR, so even though the value 42 is harmless, the TAINT BIT follows it through every computation.

DUT.IMEM.mem[19] = 32'h10702023;  // sw x7, 256(x0)

THE ATTACK: Store x7 (which is TAINTED, derived from x5) to address 256 (0x100), which is in the PROTECTED region. This is the instruction that triggers taint_exception = 1.

DUT.IMEM.mem[20] = 32'h10102023;  // sw x1, 256(x0)

SAFE STORE: Store x1 (which is CLEAN) to the SAME protected address 0x100. This proves the system has zero false positives — clean data can go anywhere.

## Reset Sequence

rst = 1; repeat(3) @(posedge clk); rst = 0; @(posedge clk);

Hold reset for 3 cycles (clears all pipeline registers and PC). Then release reset — the pipeline starts executing from address 0 (which is a NOP).

## ACT 1: CSR Configuration

write_csr(12'h803, 32'h0000_0100);

Write 0x100 (256) to CSR 0x803 (TAINT_PROT_BASE). This sets the START of the protected memory region.

write_csr(12'h804, 32'h0000_0200);

Write 0x200 (512) to CSR 0x804 (TAINT_PROT_BOUND). This sets the END of the protected region. Any store to addresses 0x100 through 0x1FF is checked for taint.

write_csr(12'h801, 32'h0000_0020);

Write 0x20 (binary: bit 5 = 1) to CSR 0x801 (TAINT_SRC_MASK). This marks register x5 as a TAINT SOURCE. The csr_taint module automatically pulses taint_init, which loads this mask into the taint register file.

write_csr(12'h800, 32'h0000_0001);

Write 1 to CSR 0x800 (TAINT_EN). This ENABLES taint tracking. Now the taint_unit will start checking every store instruction for violations.

## ACT 2: Live Taint Display

repeat(40) begin @(posedge clk); #1;

Run for 40 clock cycles. The #1 adds a tiny delay (1ns) after the clock edge so that all signals have settled before we read them.

$display("%3d | %b %b %b %b %b %b %b %b | %s",

$display prints to the console. We show: cycle number, taint bits for x7 through x0, and whether an exception occurred. This creates a live table showing taint spreading through registers cycle by cycle.

taint_exception ? ">>> VIOLATION <<<" : ""

The ternary operator '? :' checks if taint_exception is 1. If yes, print the violation message. If no, print nothing. This highlights the exact cycle where the attack is caught.

## ACT 3: Final State Display

show_reg(1,  "loaded constant 5");

Display register x1: its value (5), taint status (clean), and description. The show_reg task reads the value from DUT.RF.regs[1] and checks taint_status[1].

show_reg(5,  "ATTACKER INPUT = 42");

Display register x5: value=42, TAINTED. This is the root source of all taint in the system.

show_reg(7,  "= x6 + x1 = 48");

Display register x7: value=48, TAINTED. Even though x1 (=5) was clean, mixing it with tainted x6 (=43) produces a tainted result. This demonstrates taint propagation through multiple levels of computation.

## ACT 4: Pass/Fail Check

if (exception_count >= 1) begin

Check if at least one taint violation was caught.

$display("RESULT: PASS — ATTACK DETECTED!");

If violations were caught: the security system is working! Print PASS.

end else begin

If no violations were caught: something is wrong with the configuration.

$display("RESULT: FAIL — Check configuration");

Print FAIL. This happened earlier when the program ran before CSR config was done (fixed by adding NOP padding).

$finish;

End the simulation.

endmodule

End of the testbench module.

END OF LINE-BY-LINE CODE EXPLANATION

All 19 Verilog files covered (17 source + 2 testbench)